Installation of EasyCasting PiBox

- Introduction

- This project is a prototype

- Arduino detections by Raspberry Pi are manual

- Passwords are by defaults

- The code is not protected

- Communications are not protected

- The code is not ready for industry, it does not resist to component failures

- No GUI

- The installation is difficult

- Create the SDCard

- Download and install SDCard Formatter : https://www.sdcard.org/downloads/formatter/

- Format a 32Go micro SDCard (with adapter) with SDCard Formatter with the option Overwrite Format

- Download the Raspbian Pibox image here : www.cyrilvincent.com/download/raspbian-pibox.rar

- Dezip the file to obtain a .img

- Download and install win32diskimager: https://sourceforge.net/projects/win32diskimager/

- Put the image file on the SDCard with win32diskimager

- Eject properly de SDCard

- First boot of the Raspberry

- Put the SDCard into the Raspberry

- Plug a keyboard and a mouse on the USB

- Plug a screen on the micro-HDMI port

- Plug the Raspberry with a power supply USB –C 5V 2A

- Network

- Network is mandatory to access and debug the Raspberry from a PC

- Internet is mandatory to install drivers while the development process

- Connect to the WIFI with the Wifi icon at the bottom right of the screen

- Test Internet with the Chromium browser (first icon after de raspberry icon)

- If you have only an Ethernet connection plug it but it will be more difficult to share file

- Open a terminal with the terminal icon

- Type ifconfig

- Note your ip address in the section wlan0 / inet

- In this example the ip address is 192.168.1.104

- Remote access with Remote Desktop

- On Windows open a remote desktop

- Enter raspberrypi, if it does not work enter the raspberry ip address

- The login is pi, the password is raspberry

- You can now unplug the screen and the keyboard from the raspberry

- The code is into the pibox directory

- This documentation is into pibox/doc directory

- Never change a file into the pibox directory, except config.py

- Remote access with Samba

- You can have a remote access to a shared directory with the Samba protocol

- On your PC open a file explorer

- Type \\raspberrypi or ip address

- You should see the directory of the pi user

- This documentation is into pibox/doc directory

- Never change a file into the pibox directory, except config.py

- Make a copy of the pibox directory on your PC

- The PiBox application

- The Raspberry and Arduino applications and source code are into the pibox folder

- To stop the application just type Enter into the terminal and wait 12 seconds

- To start the application double click on Start PiBox on the desktop

- You will see a lot of error message because there is no device

- Pair the Raspberry with Windows

- On windows 10, type Bluetooth in Cortana and find the Bluetooth Parameter screen

- On Raspbian click on the Bluetooth icon

- If the Bluetooth icon does not appear, unplug the screen from the Raspberry and reboot

- Click on Make Discoverable

- Accept connections

- Pair the Raspberry with Android

- On Raspbian click on the Bluetooth icon

- Click on Make Discoverable

- On Android, into the bluetooth parameters make the phone visible and search the Raspberrypi device

- Install the Serial Bluetooth Terminal : https://play.google.com/store/apps/details?id=de.kai_morich.serial_bluetooth_terminal

- Launch de Serial Bluetooth Terminal

- Click on the 3 bars

- Choose the raspberrypi

- You should see something like this every 2s :

- {pho:0, temp:-4, pre:-4, wei:-4, mix:-4}

- -4 means device is down, try to reconnect every 10s

- -3 means the device is normally disconnected

- -2 means the device is disconnect, try to connect now

- -1 means the device is connected but no value was send

- 0 means the device is OK

- > 0 means the last value send by the device, with a timeout of 60s

- Type on your phone 4 + Send

- Youd should receive {pho:4, temp:-4, pre:-4, wei:-4, mix:-4}

- Test with other values

- The PiBox software can work with 3 sensors, 1 mixer, 1 phone

- Each sensor can be accessed with Bluetooth, Serial USB, Serial RS 232

- Install and push Arduino Software

- Download and Install the Arduino IDE : https://www.arduino.cc/en/main/software

- Plug an Arduino Uno Board with an USB B Cable on your PC or an external power supply (6V-12V 0.2A- 1A)

- Start the Arduino IDE

- In Tools menu select Card Type : Arduino / Genuino Uno

- In Port select the COM port which display Arduino

- Choose Récupérer les informations de la carte, you should see the serial number of the card

- From the raspberry pi copy the pibox/Arduino folder on your PC

- From the Arduino IDE open the file pibox/Arduino/tests/test_arduino/test_arduino.ino

- Click on the push button

- The led of Arduino should blink every second

- Open the file test_arduino_serial.ino

- Push the code

- Open Tools / Serial monitor

- Choose 9600 bauds

- You should see

- Build and configure the Arduino temperature sensor

- Take an Arduino board

- Take a Grove Base shield

- Put the little switch button (5) on the 5v position

- Plug the base on the Arduino

- Take a temperature sensor

- Connect the sensor on the A0 connector of the base shield

- Take a bluetooth device

- Connect the device on the D6 connector of the base shield

- In Arduino IDE open the file pibox/Arduino/temp_sensor_12_bt/temp_sensor_12_bt.ino

- This device work both bluetooth and serial

- Choose the correct serial port in Tools / Port

- Choose Tools / Récupérer les informations de la carte, you should see the serial number of the card

- Notice the Serial Number in the comment

- Uncomment the line //setupBlueToothConnection(); (remove the //) and save

- Push the code

- Open the serial monitor and you should see the tem:22.00 where 22.00 is the temperature in degrees

- Comment the line setupBlueToothConnection() (put // before the function) and save

- Push

- On Android add the bluetooth device tempbt with the PIN code 1234

- Open the Serial Bluetooth Terminal, choose the tempbt device, you should see the temperature

- On raspbian pair the tempbt device

- On the bluetooth menu choose devices

- Notice the MAC address of tempbt (eg. 00: 0E:EA:CF:58:B8)

- Open the file pibox/config.py (double click)

- Modifiy the tempId value with the real MAC address

- Stop the pibox software if running simply by pressing Enter, wait 12s

- Start de Pibox with  and click on Execute in terminal

- Wait

- On Android open the Serial Bluetooth Terminal and choose the raspberry device

- Wait

- You should receive {pho:-4, temp:22.00, pre:-4, wei:-4, mix:-4}

- Build and configure the Arduino weight mock

- Take an Arduino board

- Take a Bluetooth Shield

- Locate the Rx and Tx jumpers on the board

- Put Rx jumper at the 6th position and Tx jumper at the 7th

- Plug the shield on the Arduino

- In Arduino IDE open the file weight_mock_bt.ino

- This device work both bluetooth and serial

- Choose the correct serial port in Tools / Port

- Choose Tools / Récupérer les informations de la carte, you should see the serial number of the card

- Notice the Serial Number

- Uncomment the line //setupBlueToothConnection(); (remove the //) and save

- Push the code

- Open the serial monitor and you should see the wei:2500 where 2500 is the mock weight in gram (it's an iteration from 2000 to 3000 every 2s)

- Comment the line setupBlueToothConnection() (put //) and save

- Push

- On Android add the bluetooth device weightbt with the PIN code 1234

- Open the Serial Bluetooth Terminal, choose the weightbt device, you should see the temperature

- On raspbian pair the weightbt device

- On the bluetooth menu choose devices

- Notice the MAC address of weightbt (eg. 00: 0E:EA:CF:58:B8)

- Open the file pibox/config.py (double click)

- Modifiy the weightId value with the real MAC address

- Stop the pibox software if running simply by pressing Enter, wait 12s

- Start de Pibox with  and click on Execute in terminal

- Wait

- On Android open the Serial Bluetooth Terminal and choose the raspberry device

- Wait

- You should receive {pho:-4, temp:22.00, pre:-4, wei:2500, mix:-4}

- Build and configure the Arduino pressure sensor

- Take an Arduino board

- Take a Grove Base shield

- Put the little switch button (5) on the 5v position

- Plug the base on the Arduino

- Take a pressure sensor

- Connect the sensor on a I2C connector

- In Arduino IDE open the file pressure_bm280_sensor.ino

- Choose the correct serial port in Tools / Port

- Choose Tools / Récupérer les informations de la carte, you should see the serial number of the card

- Notice the Serial Number

- Push the code

- Open the serial monitor and you should see the pre:99500 where 99500 is the atmospheric pressure in Pa

- Plug the device on any USB port of the raspberry

- On the raspberry open a terminal

- Type cd /etc/udev/rules.d

- Type sudo nano 99-com.rules

- With arrows keys scroll down to the end of the file

- Find this line (before last line) : SUBSYSTEM=="tty", ATTRS{idVendor}=="2341", ATTRS{idProduct}=="0043", ATTRS{serial}=="857303139363515181D2", SYMLINK+="ttyArduinoPre"

- Replace the value in ATTRS{serial}=="857303139363515181D2" by the serial number of the Arduino board

- Verify that idVendor and idProduct are the same than the Arduino board

- Control+S to save

- Control+X to exit

- Reboot and wait

- On Android open the Serial Bluetooth Terminal and choose the raspberry device

- Wait

- You should receive {pho:-4, temp:22.00, pre:99.5, wei:2500, mix:-4}

- Pressure is now in HPa

- Build and configure the Arduino mix mock

- Take an Arduino board

- Take a Grove Base shield

- Put the little switch button (5) on the 5v position

- Plug the base on the Arduino

- Take a LCD Display

- Connect the sensor on a I2C connector

- In Arduino IDE open the file mix_mock.ino

- Choose the correct serial port in Tools / Port

- Choose Tools / Récupérer les informations de la carte, you should see the serial number of the card

- Notice the Serial Number

- Push the code

- Open the serial monitor

- Enter a number between 1 and 31 into the serial monitor (part 2 of the screen) and press the buttons end (3)

- The Arduino print mix:101010

- The sending number represent the input of the mix board, the response is a binary implementation of the device outputs, the left bit represent the state of the board (1 for ok). The 5 other bits represents the 5 outputs of the board

- Plug the device on any USB port of the raspberry

- On the raspberry open a terminal

- Type cd /etc/udev/rules.d

- Type sudo nano 99-com.rules

- With arrows keys scroll down to the end of the file

- Find this line (last line) : SUBSYSTEM=="tty", ATTRS{idVendor}=="2341", ATTRS{idProduct}=="0043", ATTRS{serial}=="857303139363515181D2", SYMLINK+="ttyArduinoMix"

- Replace the value in ATTRS{serial}=="857303139363515181D2" by the serial number of the Arduino board

- Verify that idVendor and idProduct are the same

- Control+S to save

- Control+X to exit

- Reboot the system

- Wait

- On Android open the Serial Bluetooth Terminal and choose the raspberry device

- Wait

- You should receive {pho:-4, temp:22.00, pre:99.5, wei:2500, mix:100000}

- The LCD should display -> and <-100000

- 100000 means the device is ready and the 5 ouputs are set to 0

- On the phone send a number between 1 and 31

- You should receive {pho:1, temp:22.00, pre:99.5, wei:2500, mix:100001}

- The LCD should display ->number and <-100001 the response of the device (1 bit for the device status, 5 bits for the 5 outputs)

- Plug and configure the screen

- Open and read the documentation in pibox/doc/pj2-rb-tft3-2-3-5-fr-1573.pdf

- Start automatically without screen

- The application start automatically only if a desktop (if a screen is plug) or via a remote desktop

- To start automatically the PiBox without screen and remote type sudo nano /boot/config.txt

- Uncomment the line #hdmi_force_hotplug=1

- Save & Reboot

- You can also access remotely to raspbian with X11VNC, type x11vnc

- From your PC download and install VNC Viewer

- In VNC Viewer type raspberrypi and you can connect to raspberry

- If you want to automatically start a VNC server type sudo raspi-config and serach for enable VNC server

- You can also access remotely to raspbian with SSH, simply put a empty file ssh at the root of the sd card, reboot and access with a SSH client : ssh pi@raspberrypi

- Create a Raspbian image

- Shut down the Raspberry

- Eject the SD Card

- Put it into your PC

- Start Win32 Disk Imager

- Select the device

- Create an image file

- Click Read

- RS232

- Jumpers Rx 7 Tx 6

- https://www.framboise314.fr/le-port-serie-du-raspberry-pi-3-pas-simple/

- The code

- All the code is under the pibox directory

- The main program is btpibox.py

- All raspbian scripts are into the scripts folder

- Communications between phone and raspberry can be done with Bluetooth 5, Wifi and Rest API + Websocket

- All communications between raspberry and devices can be done with Bluetooth 3, Serial UART, USB and RS 232

- The code is updated automatically at every start via Git

- Never modify a file under the pibox directory except the config.py

- Raspberry development : Python 3.7

- Raspberry scripts : bash

- Arduino development : C++ 11 + GCC 11 Arduino 1.8

- Raspberry OS : Raspbian Buster 10 based on Debian Buster 10 ARM32

- IDE Python : Pycharm 2019 CE

- IDE Arduino : Arduino IDE 1.8

- Tested on Raspian 10 and Windows 10

- Tested on Seeeduino board