Installation of EasyCasting PiBox Mock

- Create the SDCard

- Download and install SDCard Formatter : https://www.sdcard.org/downloads/formatter/

- Format a 32Go micro SDCard (with adapter) with SDCard Formatter with the option Overwrite Format

- Download the Raspbian Pibox image here : www.cyrilvincent.com/download/raspbian-pibox-mock.rar

- Dezip the file to obtain a .img

- Download and install win32diskimager: https://sourceforge.net/projects/win32diskimager/

- Put the image file on the SDCard with win32diskimager

- Eject properly de SDCard

- Sometimes this step doesn't work, it's due to the different size of the card

- We can create manually the Raspian OS look at the before last chapter : Install the Raspberry from scratch

- First boot of the Raspberry

- Put the SDCard into the Raspberry

- Plug a keyboard and a mouse on the USB

- Plug a screen on the micro-HDMI port

- Plug the Raspberry with a power supply USB – C 5.1V 2A

- Network

- Network is mandatory to access and debug the Raspberry from a PC

- Internet is mandatory to install drivers while the development process

- Connect to the WIFI with the Wifi icon at the bottom right of the screen

- Test Internet with the Chromium browser (first icon after de raspberry icon)

- If you have only an Ethernet connection plug it but it will be more difficult to share file

- Open a terminal with the terminal icon

- Type ifconfig

- Note your ip address in the section wlan0 / inet

- In this example the ip address is 192.168.1.104

- Remote access with Remote Desktop

- On Windows open a remote desktop

- Enter raspberrypi, if it does not work enter the raspberry ip address

- The login is pi, the password is raspberry

- You can now unplug the screen and the keyboard from the raspberry

- The code is into the pibox directory

- This documentation is into pibox/doc directory

- Never change a file into the pibox directory, except config.py

- Remote access with Samba

- You can have a remote access to a shared directory with the Samba protocol

- On your PC open a file explorer

- Type \\raspberrypi or ip address

- You should see the directory of the pi user

- This documentation is into pibox/doc directory

- Never change a file into the pibox directory, except config.py

- Make a copy of the pibox directory on your PC

- The PiBox application

- The Raspberry and Arduino applications and source code are into the pibox folder

- To stop the application just type Enter into the terminal and wait 12 seconds

- To start the application double click on Start PiBox on the desktop

- Pair the Raspberry with Windows

- On windows 10, type Bluetooth in Cortana and find the Bluetooth Parameter screen

- On Raspbian click on the Bluetooth icon

- If the Bluetooth icon does not appear, unplug the screen from the Raspberry and reboot

- Click on Make Discoverable

- Accept connections

- On the Bluetooth icon on the raspberry click on Devices

- You should see your PC

- Check the little star which means the device is safe (press the big yellow star)

- Pair the Raspberry with Android

- On Raspbian click on the Bluetooth icon

- Click on Make Discoverable

- On Android, into the bluetooth parameters make the phone visible and search the Raspberrypi device

- On the Bluetooth icon on the raspberry click on Devices

- You should see your phone

- Check the little star which means the device is safe (press the big yellow star)

- Install the Serial Bluetooth Terminal : https://play.google.com/store/apps/details?id=de.kai_morich.serial_bluetooth_terminal

- Launch de Serial Bluetooth Terminal

- Click on the 3 bars

- Choose the raspberrypi

- You should see something like this every 2s :

- {pho:-2, temp:20.4, pre:1000, wei:2500, mix:0}

- -4 means device is down, try to reconnect every 10s

- -3 means the device is normally disconnected

- -2 means the device is disconnect, try to connect now

- -1 means the device is connected but no value was sent

- 0 means the device is OK

- > 0 means the last value send by the device, with a timeout of 60s

- Type on your phone 4 + Send

- Youd should receive {pho:4, temp:-4, pre:-4, wei:-4, mix:100010}

- Data send by Bluetooth respects the JSON standard

- Mix data means

- -4 means device is down, try to reconnect every 10s

- -3 means the device is normally disconnected

- -2 means the device is disconnect, try to connect now

- -1 means the device is connected but no value was sent

- 0 means the device is OK but no data received

- > 1 you should receive a 6 bits binary

- The first bit = 1 means it's ok

- The 5 other bits simulate the 5 mixers's outputs

- Test with other values

- You should configure values in pibox/config.py in the hardwareConfig section and in the pibox/data directory

- The code

- All the code is under the pibox directory

- The main program is btpibox.py

- All raspbian scripts are into the scripts folder

- Communications between phone and raspberry can be done with Bluetooth 5, Wifi and Rest API + Websocket

- All communications between raspberry and devices can be done with Bluetooth 3  or USB

- The code is updated automatically at every start via Git

- Never modify a file under the pibox directory except the config.py

- Raspberry development : Python 3.7

- Raspberry scripts : bash

- Arduino development : C++ 11 + GCC 11 Arduino 1.8

- Raspberry OS : Raspbian Buster 10 based on Debian Buster 10 ARM32

- IDE Python : Pycharm 2019 CE

- IDE Arduino : Arduino IDE 1.8

- Tested on Raspian 10 and Windows 10

- Tested on Seeeduino board